Spis treści

I.	INFORMACJA NT. SPEŁNIENIA WARUNKÓW TECHNICZNYCH I FUNKCJONALNYCH PRZEDMIOTU DOSTAWY:	4

1.	Modernizowana linia CCV	4

2.	Urządzenie zdawcze	5

3.	Urządzenie odbiorcze	6

4.	Odciąg pomocniczy i licznik metrów przed urządzeniem odbiorczym	6

5.	Cewki podgrzewacza indukcyjnego żyły roboczej przed głowica	7

6.	Układ prowadzenia dla żył roboczych RM,RE,  RMC, RMS	8

7.	Modyfikacja układu plastyfikującego  dla ekranów półprzewodzących	8

8.	Głowica 3 warstwowa do izolacji kabli o średnicy nie mniejszej niż  150mm	9

9.	Separator magnetyczny w układzie podawania materiału izolacyjnego do wytłaczarki	10

10.	Skaner powierzchni układu izolacyjnego za strefą chłodzenia	11

11.	Oprogramowanie do kalkulacji kabli	12

12.	Nożyce hydrauliczne do cięcia	12

13.	Wyposażenie dodatkowe	12

14.	Materiały związane z instalacją,  uruchomieniem i testami odbiorczymi linii oraz nadzór i uruchomienie przez przedstawicieli dostawcy  linii	13

II.	CENA ZRYCZAŁTOWANA	15

III.	GWARANCJA	15

IV.	TERMIN REALIZACJI ZAMÓWIENIA	15

V.	OKREŚLENIE CZASU WAŻNOŚCI OFERTY	16

VI.	PRZESZKOLENIE PERSONELU	17

VII.	KRAJ POCHODZENIA PRZEDMIOTU DOSTAWY	17

VIII.	ZGODA NA PRZETWARZANIE DANYCH OSOBOWYCH:	18

IX.	OŚWIADCZENIA:	19

Załącznik nr 3

do zapytania ofertowego

nr 01/POIR.01.01.01-00-0005/20/2022

- modernizowana linia CCV

Wzór formularza oferty wraz z wymaganymi  oświadczeniami i specyfikacją techniczną przedmiotu dostawy

...............................................................

Miejscowość i data

FORMULARZ OFERTY

OFERENT:

| Nazwa : |  |
| --- | --- |
| Adres: |  |
| NIP (jeśli istnieje) |  |
| REGON |  |
| KRS |  |
| Osoba do kontaktu: |  |
| Stanowisko: |  |
| e-mail: |  |
| telefon: |  |

ZAMAWIAJĄCY: Tele-Fonika Kable S.A., ul. Hipolita Cegielskiego 1, 32-400 Myślenice

Szanowni Państwo,

W odpowiedzi na  zapytanie ofertowe z dnia ……… r o numerze 01/POIR.01.01.01-00-0005/20/2022- modernizacja linii CCV na  opracowanie dokumentacji, dostarczenia wyposażenia i zmodernizowania istniejącej linii izolacyjnej EPL 50 421 składającej się z następujących elementów linii  CCV w tym:

| - wytłaczarka i ślimak do ekranu półprzewodzącego, - dwa urządzenia zdawcze, |  |
| --- | --- |
| - jedno urządzenie odbiorcze, |  |
| - cewki podgrzewacza indukcyjnego, - układ prowadzenia dla żył roboczych RM, RMC, RMS, - układ separatora magnetycznego w układzie podawania materiału izolacyjnego, |  |
| - głowica 3 warstwowa do układu izolacyjnego kabli, |
| - odciąg pomocniczy 2, |
| - licznik długości, |
| - skaner powierzchni układu izolacyjnego za strefą chłodzenia, |
| - program kalkulacyjny, - materiały związane z instalacją, uruchomieniem i testami odbiorczymi linii oraz nadzór i uruchomienie przez przedstawicieli dostawcy linii. |  |  |

Składamy ofertę zgodnie z zakresem przedstawionym w zapytaniu ofertowym.

- INFORMACJA NT. SPEŁNIENIA WARUNKÓW TECHNICZNYCH I FUNKCJONALNYCH PRZEDMIOTU DOSTAWY:

- Modernizowana linia CCV

| L.p. | Funkcjonalność - Linia CCV | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
|  | Modernizacja istniejącej linii łańcuchowej zgodnie z opisem technicznym |  |
|  | Wyposażenie linii w podwójny układ zdawczy i układ odbiorczy dopasowany do dostępnej przestrzeni w hali produkcyjnej i zgodnie z opisem technicznym |  |
|  | Wytłaczanie izolacji i ekranów półprzewodzącą z materiałów XLPE zgodnie z opisem technicznym |  |
|  | Układ prowadzenia żył roboczych RM, RE, RMC, RMS |  |
|  | Dodatkowy układ kompaktujący przed głowicą wytłaczarki dla żył roboczych RMS |  |
|  | Filtrowanie materiału izolacyjnego poprzez separator magnetyczny |  |
|  | Wyposażenie do kontroli powierzchni ekranu zewnętrznego za strefą chłodzenia |  |
|  | Sterowanie linii dopasowane do zakresu modernizacji i kompatybilne do istniejącej linii |  |
|  | Modyfikacja układu plastyfikującego dla ekranów półprzewodzących |  |
|  | Doposażenie linii w głowicę do wytłaczania 3 warstwowych układów izolacyjnych |  |
|  | Oprogramowanie do kalkulacji kabli |  |

| L.p. | Parametry techniczne - Linia CCV | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
|  | Zakres przekrojów żył roboczych | Al: 150 - 3200 mm2 Cu: 150 - 3000 mm2 |  |  |  |
|  | Maksymalna średnica żyły izolowanej | do 150 mm |  |  |  |
|  | Zakres napięć dla izolacji | 10 – 500 kV |  |  |  |
|  | Ciśnienie azotu | do 15 barów |  |  |  |
|  | Maksymalna waga żyły izolowanej | do 40 kg/m |  |  |  |
|  | Zakres grubości ekranu na żyle roboczej | 0,4 - 2,5 mm |  |  |  |
|  | Zakres grubości izolacji | do 32 mm |  |  |  |
|  | Zakres grubości ekranu na izolacji | 0,4 - 2,0 mm |  |  |  |
|  | Prędkość pracy linii | 0,3 - 25 m/min |  |  |  |
|  | Kolor elementów linii: - elementów stałe - elementy ruchome - panele i szafy sterownicze | RAL 6021 RAL 1003 RAL 7035 |  |  |  |
|  | Certyfikat CE | tak |  |  |  |

- Urządzenie zdawcze

| L.p. | Funkcjonalność - Urządzenie zdawcze | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Automatyczne podnoszenie i opuszczanie bębnów |  |
| 2. | Możliwość wykonania zmiany bębna bez zatrzymywania linii |  |

| L.p. | Parametry techniczne - Urządzenie zdawcze | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Liczba urządzeń zdawczych | 2 szt. |  |  |  |
| 2. | Średnica bębna -znormalizowane normą DIN | 1800 - 5000 mm |  |  |  |
| 3. | Szerokość bębna - znormalizowane normą DIN | 1600 - 3300 mm |  |  |  |
| 4. | Nośność | do 75 ton |  |  |  |

- Urządzenie odbiorcze

| L.p. | Funkcjonalność - Urządzenie odbiorcze | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Automatyczne podnoszenie i opuszczanie bębnów |  |
| 2. | Możliwość wyboru urządzenia odbiorczego w zależności od produkowanego asortymentu i nośności |  |

| L.p. | Parametry techniczne - Urządzenie odbiorcze | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Liczba urządzeń odbiorczych | 1 szt. |  |  |  |
| 2. | Średnica bębna -znormalizowane normą DIN | 1800 - 5000 mm |  |  |  |
| 3. | Szerokość bębna - znormalizowane normą DIN | 1800 - 3900 mm |  |  |  |
| 4. | Nośność | do 90 ton |  |  |  |

- Odciąg pomocniczy i licznik metrów przed urządzeniem odbiorczym

| L.p. | Funkcjonalność - Odciąg pomocniczy i licznik metrów przed urządzeniem | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Odciąg gąsienicowy dostosowany do parametrów linii |  |
| 2. | Dodatkowy licznik metrów dopasowany do linii |  |
| 3. | Wymagane sterowanie i integracja w/w z istniejącą linią CCV |  |

| L.p. | Parametry techniczne - Odciąg pomocniczy i licznik metrów przed urządzeniem | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Odciąg pomocniczy | 1 komplet |  |  |  |
| 2. | Licznik metrów gąsienicowy | 1 komplet |  |  |  |

- Cewki podgrzewacza indukcyjnego żyły roboczej przed głowica

| L.p. | Funkcjonalność - Układ indukcyjnego podgrzewacza żyły roboczej przed głowica | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Średnice cewek odpowiednie dla zakresu przekrojów żył roboczych |  |

| L.p. | Parametry techniczne - Układ indukcyjnego podgrzewacza żyły roboczej przed głowica | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Liczba cewek | 3 szt. |  |  |  |
| 2. | Zakres średnic cewek | 630 -1000 mm2 1200 -2000 mm2 >2000 - 3200 mm2 |  |  |  |

- Układ prowadzenia dla żył roboczych RM,RE,  RMC, RMS

| L.p. | Funkcjonalność - Układ prowadzenia dla żył roboczych RM, RMC, RMS | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Układ kompaktujący dla żyły RMS przed głowicą |  |
| 2. | Układ prowadzący żyłę roboczą centralnie w głowicy |  |

| L.p. | Parametry techniczne - Układ prowadzenia dla żył roboczych RM, RMC , RMS, RE | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Zakres układu kompaktującego dla żył RMS | Al/Cu 800-3200 mm2 |  |  |  |

- Modyfikacja układu plastyfikującego  dla ekranów półprzewodzących

| L.p. | Funkcjonalność - Modyfikacja układu plastyfikującego dla ekranów półprzewodzących | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Wymiana wytaczarki ekranu półprzewodzącego |  |
| 2. | Wytłaczarka wyposażona w układy grzewczo chłodzące elektryczno-wodne (chłodzenie wodne wytłaczarki) |  |
| 3. | Możliwość odstawiania wytłaczaki do tyłu |  |
| 4. | Dostosowanie istniejącego układu zasypu granulatu do nowej wytłaczarki |  |
| 5. | Ślimak do wytłaczarki ekranu zewnętrznego o nowej geometrii |  |

| L.p. | Parametry techniczne - Układ wytłaczarek poziomych | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Liczba wytłaczarek do wymiany | 1 szt. |  |  |  |
| 2. | Średnica cylindra wytłaczarki dla ekranu półprzewodzącego | min. 80 mm |  |  |  |
| 3. | Długość cylindra wytłaczarki | min. 20D |  |  |  |
| 4. | Wydajność wytłaczarki | 20 - 60 kg/h |  |  |  |
| 5. | Nowy ślimak do wytłaczarki ekranu zewnętrznego | 80 mm |  |  |  |

- Głowica 3 warstwowa do izolacji kabli o średnicy nie mniejszej niż  150mm

| L.p. | Funkcjonalność - Głowica 3 warstwowa do izolacji kabli o średnicy nie mniejszej niż 150mm | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Głowica wytłaczająca układ izolacyjny jednooperacyjnie: ekran na żyle, izolacja, ekran na izolacji lub opcjonalnie ekran na żyle, izolacja |  |
| 2. | Ręczne centrowanie dla każdej warstwy |  |
| 3. | Łatwy demontaż łączników wytłaczarki |  |
| 4. | Ergonomiczna i bezpieczna wymiana głowicy |  |
| 5. | Szybkie czyszczenie głowicy |  |
| 6. | Oprzyrządowanie do wytłaczania: 2 zestawy do testów prototypowego kabla |  |
| 7. | Hydrauliczne wysuwanie rozdzielacza (opcjonalnie) |  |
| 8. | Ergonomiczna i bezpieczna wymiana oprzyrządowania |  |

| L.p. | Parametry techniczne - Głowica 3 warstwowa do izolacji kabli o średnicy nie mniejszej niż 150mm | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Liczba głowic | 1 szt. |  |  |  |
| 2. | Średnica na wejściu do głowicy | 20 - 90 mm |  |  |  |
| 3. | Zakres grubości wytłaczanej izolacji | 4 – 35 mm |  |  |  |
| 4. | Głowica musi posiadać grzanie wodne lub wodno - elektryczne | 40 - 130°C |  |  |  |
| 5. | Głowica musi posiadać odczyt temperatury z poszczególnych elementów | Zestaw czujników |  |  |  |

- Separator magnetyczny w układzie podawania materiału izolacyjnego do wytłaczarki

| L.p. | Funkcjonalność - Separator magnetyczny w układzie podawania materiału izolacyjnego do wytłaczarki | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Oddzielanie zanieczyszczeń metalicznych z granulatu izolacyjnego |  |
| 2. | Separator magnetyczny przystosowany do grawitacyjnego systemu podawania zainstalowanego na linii CCV nad lejem zasypowym wytłaczarki |  |
| 3. | Łatwe w czyszczeniu elementy magnetyczne |  |
| 4. | Części ze stali nierdzewnej z polerowanym wykończeniem wewnątrz i na zewnątrz |  |
| 5. | Wymagana integracja z istniejącą linią CCV |  |

| L.p. | Parametry techniczne - Układ podawania materiału izolacyjnego do wytłaczarki z separatorem powietrznym i magnetycznym | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Wydajność separatora | Do 600 kg/h |  |  |  |
| 2. | Usuniecie zanieczyszczeń metalicznych z materiału | Do 100% |  |  |  |

- Skaner powierzchni układu izolacyjnego za strefą chłodzenia

| L.p. | Funkcjonalność - Skaner powierzchni układu izolacyjnego | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Sprawdzanie stanu powierzchni układu izolacyjnego za strefa chłodzenia |  |
| 2. | System umożliwiający zapis i przesyłania wszystkich danych z przyrządów pomiarowych w czasie rzeczywistym |  |
| 3. | Poprawne działanie urządzenia od początku odcinka produkcyjnego do jego zakończenia |  |
| 4. | Wymagane sterowanie, integracja z istniejąca linia CCV oraz wizualizacja parametrów w sterowni linii |  |

| L.p. | Parametry techniczne - System pomiaru geometrii na gorąco/zimno | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Pomiar średnicy i geometrii układu izolacyjnego "na zimno": a) Średnica na ekranie izolacji | do 155 mm |  |  |  |
| 2. | System umożliwiający zapis i przesyłania wszystkich danych z przyrządów pomiarowych w czasie rzeczywistym | komplet |  |  |  |

- Oprogramowanie do kalkulacji kabli

| L.p. | Funkcjonalność - program kalkulacyjny | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Program kalkulacyjny do przygotowania receptur dla kabli HV, EHV AC/DC |  |

- Nożyce hydrauliczne do cięcia

| L.p. | Funkcjonalność - Nożyce hydrauliczne do cięcia | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Możliwość cięcia wykonanego produktu |  |

| L.p. | Parametry techniczne - Nożyce hydrauliczne do cięcia | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Nożyce do cięcia w zakresie średnic 0 – 150 mm wraz z pompą i wózkiem do przemieszczania | 1 szt. |  |  |  |
| 2. | Żuraw do zawieszenia nożyc hydraulicznych | 1 szt. |  |  |  |

- Wyposażenie dodatkowe

| L.p. | Funkcjonalność - Wyposażenie dodatkowe | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Popychacze do toczenia bębnów |  |
| 2. | Praska hydrauliczna do łączenia żył roboczych z głowicą |  |

| L.p. | Parametry techniczne - Wyposażenie dodatkowe | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Popychacze 100 t | 1 szt. 100 ton lub zamiennie 2 szt. 50 t |  |  |  |
| 2. | Praska hydrauliczna | 1 szt. |  |  |  |
| 3. | Zakres żył roboczych | Al /Cu 800 – 3000 mm2 |  |  |  |
| 4. | Matryce do głowicy | komplet |  |  |  |

- Materiały związane z instalacją,  uruchomieniem i testami odbiorczymi linii oraz nadzór i uruchomienie przez przedstawicieli dostawcy  linii

| L.p. | Funkcjonalność - Materiały związane z instalacją, uruchomieniem i testami odbiorczymi linii oraz nadzór i uruchomienie przez przedstawicieli dostawcy linii | Informacja o spełnieniu danej funkcjonalności poprzez wpisanie TAK lub NIE |
| --- | --- | --- |
| 1. | Komplet wymaganego okablowania |  |
| 2. | Szafy sterownicze i zasilające do dostarczonych urządzeń |  |
| 3. | Pozostałe materiały konieczne do montażu i uruchomienia zmodernizowanej linii |  |
| 4. | Komplet oprzyrządowania na wytypowane kable odbiorowe |  |
| 5. | Nadzór dostawcy w trakcie instalacji i uruchomienie linii |  |

| L.p. | Parametry techniczne - Materiały związane z instalacją, uruchomieniem i testami odbiorczymi linii oraz nadzór i uruchomienie przez przedstawicieli dostawcy linii | Wartość | Informacja o spełnieniu wymagania (wskazać czy wartość spełnia stawiany warunek poprzez wpisanie TAK lub NIE) | Informacja o spełnieniu wymagania (podać wartość parametru oferowanego urządzenia) | Uwagi (wpisać jeśli dotyczy) |
| --- | --- | --- | --- | --- | --- |
| 1. | Komplet okablowania Kable sterownicze Kable zasilające Kable teleinformatyczne Kable specjalistyczne i złącza potrzebne do podłączenia i uruchomienia elementów linii po zmodernizowaniu | 1 komplet 1 komplet 1 komplet 1 komplet |  |  |  |
| 2. | Szafy sterownicze i zasilające do dostarczonych urządzeń Szafy zasilające Szafy sterownicze | 1 komplet 1 komplet |  |  |  |
| 3. | Niezbędne materiały potrzebne do montażu i uruchomienia linii | 1 komplet |  |  |  |
| 4. | Komplet oprzyrządowania na wytypowane kable odbiorowe | 2 komplety |  |  |  |
| 5. | Nadzór dostawcy w trakcie instalacji i uruchomienie linii a. nadzór podczas instalacji b. synchronizacja i uruchomienie linii c. testy odbiorcze i szkolenie personelu Zamawiającego d. koszty związane z zakwaterowaniem i transportem personelu Dostawcy | min. 5 dni min. 20 dni min. 5 dni 1 komplet |  |  |  |

- CENA ZRYCZAŁTOWANA

| Przedmiot zamówienia | Cena zryczałtowana netto - waluta | Cena zryczałtowana brutto – waluta (jeśli dotyczy) |
| --- | --- | --- |
| Opracowanie dokumentacji, dostarczenia wyposażenia i zmodernizowania istniejącej linii izolacyjnej EPL 50 421 składającej się z następujących elementów linii CCV w tym: - wytłaczarka i ślimak do ekranu półprzewodzącego, - dwa urządzenia zdawcze,. - jedno urządzenie odbiorcze,. - cewki podgrzewacza indukcyjnego, - układ prowadzenia dla żył roboczych RM, RMC, RMS, - układ separatora magnetycznego w układzie podawania materiału, izolacyjnego. - głowica 3 warstwowa do układu izolacyjnego kabli,. - odciąg pomocniczy 2,. - licznik długości,. - skaner powierzchni układu izolacyjnego za strefą chłodzenia. - program kalkulacyjny, - materiały związane z instalacją, uruchomieniem i testami odbiorczymi linii oraz nadzór i uruchomienie przez przedstawicieli dostawcy linii. | ….. - …. (cena- waluta) | ….. - …. (cena- waluta) |

- GWARANCJA

| Przedmiot zamówienia | Okres gwarancji (w pełnych miesiącach ) |
| --- | --- |
| Opracowanie dokumentacji, dostarczenia wyposażenia i zmodernizowania istniejącej linii izolacyjnej EPL 50 421 składającej się z następujących elementów linii CCV w tym: - wytłaczarka i ślimak do ekranu półprzewodzącego, - dwa urządzenia zdawcze,. - jedno urządzenie odbiorcze,. - cewki podgrzewacza indukcyjnego, - układ prowadzenia dla żył roboczych RM, RMC, RMS, - układ separatora magnetycznego w układzie podawania materiału, izolacyjnego. - głowica 3 warstwowa do układu izolacyjnego kabli,. - odciąg pomocniczy 2,. - licznik długości,. - skaner powierzchni układu izolacyjnego za strefą chłodzenia. - program kalkulacyjny, - materiały związane z instalacją, uruchomieniem i testami odbiorczymi linii oraz nadzór i uruchomienie przez przedstawicieli dostawcy linii. | …. (w miesiącach, podać ilość miesięcy) |

- TERMIN REALIZACJI ZAMÓWIENIA

| Przedmiot zamówienia | Przewidywany termin realizacji zamówienia od momentu podpisania umowy (w miesiącach) (termin realizacji zamówienia powinien uwzględniać cały przedmiot zamówienia zgodnie z opisem w części I zapytania ofertowego, tj. wraz projektem, dostawą, uruchomieniem i odbiorem) |
| --- | --- |
| Opracowanie dokumentacji, dostarczenia wyposażenia i zmodernizowania istniejącej linii izolacyjnej EPL 50 421 składającej się z następujących elementów linii CCV w tym: - wytłaczarka i ślimak do ekranu półprzewodzącego, - dwa urządzenia zdawcze,. - jedno urządzenie odbiorcze,. - cewki podgrzewacza indukcyjnego, - układ prowadzenia dla żył roboczych RM, RMC, RMS, - układ separatora magnetycznego w układzie podawania materiału, izolacyjnego. - głowica 3 warstwowa do układu izolacyjnego kabli,. - odciąg pomocniczy 2,. - licznik długości,. - skaner powierzchni układu izolacyjnego za strefą chłodzenia. - program kalkulacyjny, - materiały związane z instalacją, uruchomieniem i testami odbiorczymi linii oraz nadzór i uruchomienie przez przedstawicieli dostawcy linii. | - Wykonanie projektu modernizowanej linii CCV ………….. 2) Dostawa nowych komponentów modernizowanej linii CCV: ………….. 3) Uruchomienie i odbiór modernizowanej linii CCV: …………… (w miesiącach, podać liczbę miesięcy) |

- OKREŚLENIE CZASU WAŻNOŚCI OFERTY

| Okres ważności oferty (w dniach) | ( w dniach, podać ilość dni ) …………. |
| --- | --- |

- PRZESZKOLENIE PERSONELU

| Przedmiot zamówienia | Czy podana cena przedmiotu zamówienia zakłada przeszkolenie personelu |
| --- | --- |
| Opracowanie dokumentacji, dostarczenia wyposażenia i zmodernizowania istniejącej linii izolacyjnej EPL 50 421 składającej się z następujących elementów linii CCV w tym: - wytłaczarka i ślimak do ekranu półprzewodzącego, - dwa urządzenia zdawcze,. - jedno urządzenie odbiorcze,. - cewki podgrzewacza indukcyjnego, - układ prowadzenia dla żył roboczych RM, RMC, RMS, - układ separatora magnetycznego w układzie podawania materiału, izolacyjnego. - głowica 3 warstwowa do układu izolacyjnego kabli,. - odciąg pomocniczy 2,. - licznik długości,. - skaner powierzchni układu izolacyjnego za strefą chłodzenia. - program kalkulacyjny, - materiały związane z instalacją, uruchomieniem i testami odbiorczymi linii oraz nadzór i uruchomienie przez przedstawicieli dostawcy linii. | TAK NIE |

- KRAJ POCHODZENIA PRZEDMIOTU DOSTAWY

| Przedmiot zamówienia | Kraj pochodzenia przedmiotu dostawy |
| --- | --- |
| Opracowanie dokumentacji, dostarczenia wyposażenia i zmodernizowania istniejącej linii izolacyjnej EPL 50 421 składającej się z następujących elementów linii CCV w tym: - wytłaczarka i ślimak do ekranu półprzewodzącego, - dwa urządzenia zdawcze,. - jedno urządzenie odbiorcze,. - cewki podgrzewacza indukcyjnego, - układ prowadzenia dla żył roboczych RM, RMC, RMS, - układ separatora magnetycznego w układzie podawania materiału, izolacyjnego. - głowica 3 warstwowa do układu izolacyjnego kabli,. - odciąg pomocniczy 2,. - licznik długości,. - skaner powierzchni układu izolacyjnego za strefą chłodzenia. - program kalkulacyjny, - materiały związane z instalacją, uruchomieniem i testami odbiorczymi linii oraz nadzór i uruchomienie przez przedstawicieli dostawcy linii. | … ( wskazać nazwę kraju ) |

- ZGODA NA PRZETWARZANIE DANYCH OSOBOWYCH:

Niniejszym oświadczam, że:

- zapoznałem/łam się z klauzurą informacyjną opisaną poniżej

TAK,  NIE

- wyrażam zgodę na przetwarzanie danych osobowych w zakresie wymienionym poniżej

TAK,  NIE

W odniesieniu do danych osobowych Tele-Fonika Kable S.A. informuje, że Administratorem danych jest TELE-FONIKA Kable S.A. z siedzibą w Myślenicach; 32-400, przy ul. Hipolita Cegielskiego 1 (dalej również jako „Spółka”). Z administratorem można się skontaktować poprzez adres email daneosobowe@tfkable.com. Z Administratorem danych można się kontaktować we wszystkich sprawach dotyczących przetwarzania danych osobowych oraz korzystania z praw związanych z przetwarzaniem danych. Pani/Pana dane będą przetwarzane w celu odpowiedzi na przesłane zapytania – podstawą prawną przetwarzania danych jest niezbędność przetwarzania do realizacji prawnie uzasadnionego interesu administratora. Uzasadnionym interesem administratora jest prowadzenie i obsługa sprzedaży swoich wyrobów i usług, prowadzenie i obsługa zakupów towarów i usług oraz możliwość przedstawiania klientowi informacji o swoich usługach, przedstawiania ofert dopasowanych do potrzeb i zainteresowań klienta, zwiększenie sprzedaży swoich usług. Pani/Pana dane osobowe mogą być udostępniane:

- podmiotom przetwarzającym dane osobowe na zlecenie administratora (m.in. dostawcom usług IT) – przy czym takie podmioty przetwarzają dane na podstawie umowy z administratorem i wyłącznie z polecenia administratora;

- podmiotom udzielającym lub rozliczającym dofinansowanie ze środków publicznych;

- podmiotom prowadzącym działalność doradczą, podmiotom prowadzącym działalność audytorską;

- innym administratorom danych uprawnionym do uzyskania danych na podstawie obowiązującego prawa, w tym Ministerstwu Inwestycji i Rozwoju jako administratora danych w centralnym systemie informatycznych SL 2014.

Pani/Pana dane osobowe będą przechowywane do momentu wygaśnięcia obowiązku przechowywania danych wynikającego z przepisów prawa, w tym przepisów o pomocy publicznej. Przysługuje Pani/Panu prawo dostępu do Pani/Pana danych oraz prawo żądania ich sprostowania, ich usunięcia lub ograniczenia ich przetwarzania. Na Pani/Pana wniosek administrator dostarczy kopię danych osobowych podlegających przetwarzaniu, przy czym za wszelkie kolejne kopie, o które się Pani/Pan zwróci administrator może pobrać opłatę w rozsądnej wysokości wynikającej z kosztów administracyjnych. Ma Pani/Pan prawo wycofania zgody na przetwarzanie Pani/Pana danych osobowych. Wycofanie zgody nie ma wpływu na zgodność z prawem przetwarzania, którego dokonano na podstawie zgody przed jej wycofaniem. W zakresie, w jakim Pani/Pana dane są przetwarzane w sposób zautomatyzowany w celu zawarcia i wykonywania umowy lub przetwarzane na podstawie zgody – przysługuje Pani/Panu także prawo do przenoszenia danych osobowych, tj. do otrzymania od administratora Pani/Pana danych osobowych, w ustrukturyzowanym, powszechnie używanym formacie nadającym się do odczytu maszynowego. Może Pani/Pan przesłać te dane innemu administratorowi danych. Przysługuje Pani/Panu również prawo wniesienia skargi do organu nadzorczego zajmującego się ochroną danych osobowych. W celu skorzystania z powyższych praw należy skontaktować się z administratorem danych. Dane kontaktowe wskazane są wyżej. Podanie danych osobowych do wyżej wskazanych celów jest dobrowolne.

Oferent oświadcza, że zapoznał się z klauzurą informacyjną dla Oferentów i wyraża zgodę na przetwarzanie danych osobowych poprzez potwierdzenie powyższego w formularzu ofertowym stanowiącym Załącznik nr 2 do zapytania ofertowego.

- OŚWIADCZENIA:

- OŚWIADCZAMY, że zapoznaliśmy się z treścią zapytania ofertowego oraz wyjaśnieniami i zmianami  zapytania ofertowego przekazanymi przez Zamawiającego i uznajemy się za związanych określonymi w nich postanowieniami i zasadami postępowania.

- AKCEPTUJEMY warunki płatności określone przez Zamawiającego w zapytaniu ofertowym.

- JESTEŚMY związani ofertą przez czas wskazany w zapytaniu ofertowym, jednak nie krótszy niż 90 dni.

- ZOBOWIĄZUJEMY się do dostarczenia gwarancji należytego wykonania umowy i rękojmi zgodnie z wymaganiami opisanymi w zapytaniu ofertowym.

- Prezentowane w ofercie urządzenia są NOWE i SPEŁNIAJĄ wymagania wszystkich aktów normatywnych dopuszczających je do użytkowania w Polsce.

- JESTEŚMY producentem lub przedstawicielem producenta przedmiotu zamówienia  opisanym w punkcie I zapytania ofertowego. Na prośbę Zamawiającego zobowiązujemy się dostarczyć dokumenty potwierdzające współpracę z producentem umożliwiającą dostarczenie przedmiotu zamówienia.

- OŚWIADCZAMY, że przedmiot oferty nie jest obarczony wadami prawnymi i udziałem osób trzecich.

- Do oferty DOŁĄCZAMY specyfikację techniczną przedmiotu dostawy.

...................................................

Podpis Oferenta

Table of contents

I.	INFORMATION REGARDING MEETING TECHNICAL AND FUNCTIONALITY CONDITIONS OF THE SUBJECT MATTER OF THE CONTRACT:	23

1.	Upgraded CCV Line	23

2.	Pay off device	24

3.	Take-up devices	25

4.	Auxiliary caterpillar and length meter counter before the take-up	25

5.	Induction heater system for the conductor before the crosshead	26

6.	Guiding system for RM, RE, RMC, RMS conductors	27

7.	Modification of the plastication system for the semi-con screens	27

8.	A 3-layer crosshead for cable insulation of not less than 150mm in diameter	28

9.	Magnetic separator in the insulating material feeding system into the extruder	29

10.	Insulation system surface scanner behind the cooling section	30

11.	Cable calculating software	31

12.	Hydraulic cable cutters	31

13.	Additional equipment	32

14.	Materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier	33

II.	FLAT-PRICE	34

III.	WARRANTY	35

IV.	TIME OF ORDER DELIVERY	35

V.	TIME OF FAILURE REMOVAL IN TERMS OF ELECTRONICS AND INSTRUMENTATION	36

VI.	SPECIFY VALIDITY TIME OF THE PROPOSAL	36

VII.	STAFF TRANING	36

VIII.	COUNTRY OF ORIGIN OF THE DELIVERY ITEM	37

IX.	CONSENT FOR PROCESSING PERSONAL DATA:	38

X.	REPRESENTATIONS:	39

Annex no. 2

to the Request or Proposal no.:

01/POIR.01.01.01-00-0005/20/2022

upgraded CCV Line,

A model form of Proposal including required declarations and statements, together with technical specification of the Subject Matter of the Contract

...............................................................

Place and date

PROPOSAL FORM

TENDERER:

| Name : |  |
| --- | --- |
| Address: |  |
| Tax Id. No. (NIP) (if any) |  |
| VAT |  |
| Contact person: |  |
| Position: |  |
| e-mail address: |  |
| Telephone no.: |  |

CONTRACTING ENTITY: Tele-Fonika Kable S.A., ul. Hipolita Cegielskiego 1, 32-400 Myślenice

Dear Sir/ Madam,

In response to the Request for Proposal of ……… (date), Request No.: 01/POIR.01.01.01-00-0005/20/2022 - upgraded CCV Line  for the development of the documentation, delivery of the equipment and upgrading of the existing EPL 50 421 CCV line.  The upgrade shall consist of the following components:

| - extruder and screw for the semi-conductive screen, - two pay-off devices, |  |
| --- | --- |
| - one take-up device, |  |
| - induction preheater coil, - guiding system for RM, RMC, RMS conductors, - magnetic separator system in installed in the feeding and handling system for insulating material |  |
| - triple crosshead for 3 layers of cable insulating system, |
| - auxiliary caterpillar 2, |
| - length meter counter, |
| - insulation system surface scanner behind the cooling section, |
| - calculating programme, - materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier. |  |  |

We submit a proposal in accordance with the scope presented in the Request for Proposal.

.

- INFORMATION REGARDING MEETING TECHNICAL AND FUNCTIONALITY CONDITIONS OF THE SUBJECT MATTER OF THE CONTRACT:

- Upgraded CCV Line

| Item | Functionality - CCV Line | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Upgrade of the existing CCV line in accordance with the technical specification |  |
| 2. | Equipping the line with the double pay-off and take-up devices adapted to the available area in the production building, meeting the technical description |  |
| 3. | Extrusion of insulation and semi-conductive screens made of XLPE materials, in accordance with the technical description. |  |
| 4. | Guiding system for RM, RE, RMC, RMS conductors |  |
| 5. | Additional compacting system before the extrusion crosshead for RMS conductors |  |
| 6. | Possibility to filter insulating material through magnetic separator |  |
| 7. | Equipment for outer semi-con screen surface monitoring installed behind the cooling section |  |
| 8. | Line control system adapted to the scope of the upgrade and compatible with the existing line |  |
| 9. | Modification of the semi-con screens plastication system |  |
| 10. | Equipping the line with the crosshead for extrusion of 3-layer insulation systems |  |
| 11. | Calculating software |  |

| Item | Technical parameters - CCV line | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Range of conductor cross-sections | Al: 150 -3200 mm2 Cu: 150- 3000 mm2 |  |  |  |
| 2. | Maximum diameter of insulated core | up to 150 mm |  |  |  |
| 3. | Voltage range for insulation | 10 – 500 kV |  |  |  |
| 4. | Nitrogen pressure | Up to15 bar |  |  |  |
| 5. | Maximum weight of an insulated core | up to 40 kg/m |  |  |  |
| 6. | Thickness range for conductor screen | 0,4 - 2,5 mm |  |  |  |
| 7. | Thickness range for insulation | up to 32 mm |  |  |  |
| 8. | Thickness range for insulation screen | 0,4 - 2,0 mm |  |  |  |
| 9. | Line speed | 0,3 - 25 m/min |  |  |  |
| 10. | Line colour - fixed elements - moving parts - control panels and cabinets | RAl 6021 RAl 1003 RAl 7035 |  |  |  |
| 11. | CE Certificate | yes |  |  |  |

- Pay off device

| Item | Functionality – Pay-off Device | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Automatic lifting and lowering of the drums |  |
| 2. | Enabling drum change without the need of line stop |  |

| Item | Technical parameters – Pay-off device | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Number of pay-off devices | 2 pieces |  |  |  |
| 2. | Drum diameter -standardized by DIN | 1800 - 5000 mm |  |  |  |
| 3. | Drum width - standardized by DIN | 1600 - 3300 mm |  |  |  |
| 4. | Carrying capacity | up to 75 tons |  |  |  |

- Take-up devices

| Item | Functionality – Take up devices | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Automatic lifting and lowering of the drums |  |
| 2. | Possibility of device selection depending on the type of cable produced and lifting capacity of the device |  |

| Item | Technical parameters – Take up devices | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Number of take-up devices | 1 piece |  |  |  |
| 2. | Drum diameter -standardized by DIN | 1800 - 5000 mm |  |  |  |
| 3. | Drum width - standardized by DIN | 1800 - 3900 mm |  |  |  |
| 4. | Carrying capacity | up to 90 ton |  |  |  |

- Auxiliary caterpillar and length meter counter before the take-up

| Item | Functionality – Auxiliary caterpillar and length meter counter before the take-up | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Caterpillar capstan appropriate for line parameters |  |
| 2. | Additional length meter counter appropriate for the line |  |
| 3. | Required control system integrated with the existing CCV line |  |

| Item | Technical parameters – Auxiliary caterpillar and length meter counter before the take-up | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Auxiliary caterpillar | 1 set |  |  |  |
| 2. | Caterpillar-type length metre counter | 1 set |  |  |  |

- Induction heater system for the conductor before the crosshead

| Item | Functionality – Induction heater system for conductor before the crosshead | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Coil diameters suitable for the entire range of cross-sections of conductors |  |

| Item | Technical parameters – Induction heater system for the conductor before the crosshead | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Number of coils | 3 pieces |  |  |  |
| 2. | Coil diameter range | 630 -1000 mm2 1200-2000 mm2 > 2000 - 3200 mm2 |  |  |  |

- Guiding system for RM, RE, RMC, RMS conductors

| Item | Functionality – Guiding system for RM, RE, RMC, RMS conductors | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Compacting system for RMS conductor before the crosshead |  |
| 2. | The system for running the conductor in the centreline of the crosshead |  |

| Item | Technical parameters – Guiding system for RM, RE, RMC, RMS conductors | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Range of RMS conductors compacting system | Al/Cu 800 -3200 mm2 |  |  |  |

- Modification of the plastication system for the semi-con screens

| Item | Functionality – Modification of the plastication system for the semi-con screens | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Replacement of the extruder for the semi-con screen |  |
| 2. | The extruder to be equipped electric and water heating and cooling systems (extruder water cooling system) |  |
| 3. | Possibility to move extruders backwards |  |
| 4. | Necessity to adapt existing pellet feeding system to the new extruder. |  |
| 5. | The screw for the outer semi-con screen extruder with new geometry |  |

| Item | Technical parameters – System of horizontal extruders | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Number of extruders to be replaced | 1 piece |  |  |  |
| 2. | Extruder barrel diameter for the semi-conductive screen | min. 80 mm |  |  |  |
| 3. | Length of extruder barrel | min. 20D |  |  |  |
| 4. | Extruder efficiency/capacity | 20 - 60 kg/h |  |  |  |
| 5. | New screw for the outer semi-con screen extruder | 80 mm |  |  |  |

- A 3-layer crosshead for cable insulation of not less than 150mm in diameter

| Item | Functionality – A triple crosshead for insulation of at least 150mm in diameter | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Crosshead for extrusion of a three-layer insulation system in one operation: conductor screen, insulation, insulation screen or optional conductor screen, insulation |  |
| 2. | Manual centring for each layer |  |
| 3. | Easy disassembly of extruder fittings/connectors |  |
| 4. | Ergonomic and safe crosshead change |  |
| 5. | Fast crosshead cleaning |  |
| 6. | Tooling for extrusion: 2 sets for a prototype cable trial production |  |
| 7. | Hydraulic extension of the distributor (optional) |  |
| 8. | Ergonomic and safe tooling change |  |

| Item | Technical parameters – A triple crosshead for insulation of at least 150mm in diameter | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Number of crossheads | 1 piece |  |  |  |
| 2. | Diameter at the entrance of the crosshead | 20 - 90 mm |  |  |  |
| 3. | Thickness range of extrusion of insulation | 4 – 35 mm |  |  |  |
| 4. | The crosshead must be equipped with water or water-electric heating | 40- 130°C |  |  |  |
| 5. | The crosshead must be equipped with the system for temperature measurement of individual components | set of sensors |  |  |  |

- Magnetic separator in the insulating material feeding system into the extruder

| Item | Functionality – Magnetic separator to be installed in the extruder feeding system for the insulating material | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Separation of metallic contaminants from the insulating material (granulate) |  |
| 2. | Magnetic separator must be adapted to the gravitational feeding system installed over the extruder hopper of the CCV line |  |
| 3. | Magnetic components must be easy to clean |  |
| 4. | Stainless steel parts with polished finishing of the inner and outer surface |  |
| 5. | Required integration with the existing CCV line |  |

| Item | Technical parameters – Magnetic separator installed in the insulation feeding system to the extruder | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Separator capacity | up to 600 kg/h |  |  |  |
| 2. | Removal of metallic impurities from the material | up to 100% |  |  |  |

- Insulation system surface scanner behind the cooling section

| Item | Functionality – Surface scanner for insulation system installed behind the cooling section | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Monitoring of the insulation system surface quality behind the cooling section |  |
| 2. | The system must enable real-time data recording and sending from the measuring devices |  |
| 3. | Appropriate operation of the device from the start of the production length to its completion. |  |
| 4. | Required control system integrated with the existing CCV and the possibility to show the parameters in the line control room |  |

| Item | Technical parameters – Hot / cold geometry measurement system | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Measurement of the diameter and geometry of the "cold" insulation system: a) Diameter over the insulation screen | up to 155 mm |  |  |  |
| 2. | system that enables recording and transmitting all data from measuring instruments in real time | set |  |  |  |

- Cable calculating software

| Item | Functionality – Calculation program | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Calculating software for preparation recipes of HV, EHV AC/DC cables |  |

- Hydraulic cable cutters

| Item | Functionality – Hydraulic cable cutters | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Possibility of cutting the product |  |

| Item | Technical parameters – Hydraulic cable cutters | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Cable cutters covering the diameter range of 0-150 mm with the pump and transport trolley | 1 piece |  |  |  |
| 2. | Jib crane for hanging cable cutters | 1 piece |  |  |  |

- Additional equipment

| Item | Functionality – Additional equipment | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Cable drum pushers |  |
| 2. | Hydraulic press with a head for connecting conductors |  |

| Item | Technical parameters – Additional equipment | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | 100 t Drum pusher | 1 piece for 100 ton or alternatively 2 pieces -50 t each |  |  |  |
| 2. | Hydraulic press | 1 piece |  |  |  |
| 3. | Range of conductor cross-sections | Al /Cu 800-3000 mm2 |  |  |  |
| 4. | Dies for the head | set |  |  |  |

=

- Materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier

| Item | Functionality – Materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier | Information on meeting required functionality enter YES or NO |
| --- | --- | --- |
| 1. | Complete wiring and cabling |  |
| 2. | Control and power cabinets necessary for the supplied equipment |  |
| 3. | Other materials necessary for the assembly and commissioning of the line |  |
| 4. | A set of accessories for selected acceptance cables |  |
| 5. | Supplier supervision during installation, line start-up to be performed by the supplier |  |

| Item | Technical parameters – Materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier | Value | Information on meeting the requirement (declare whether the value meets the requirement by entering YES or NO) | Information on meeting the requirement (enter the parameter value of the offered item) | Comments (enter if applicable) |
| --- | --- | --- | --- | --- | --- |
| 1. | Complete wiring Control cables Power cables ICT cables Specialized cables and connectors needed to connect and run the line after the upgrade | 1 set 1 set 1 set 1 set |  |  |  |
| 2. | Control and power cabinets for the supplied equipment Power cabinets Control cabinets | 1 set 1 set |  |  |  |
| 3. | Materials necessary for the line assembly and the start-up | 1 set |  |  |  |
| 4. | Complete tooling for selected acceptance cables | 2 sets |  |  |  |
| 5. | Supplier supervision during installation and commissioning of the line a. supervision during installation b. synchronization and commissioning of the line c. acceptance tests and training of the customer personnel d. costs related to the accommodation and transport of the Supplier's personnel | min. 5 days min. 20 days min. 5 days 1 set |  |  |  |

- FLAT-PRICE

| Subject Matter of the Contract | Net flat-rate price - currency | Gross flat-rate price (if applicable) |
| --- | --- | --- |
| for the development of the documentation, delivery of the equipment and upgrading of the existing EPL 50 421 CCV line. The upgrade shall consist of the following components: - extruder and screw for the semi-conductive screen, - two pay-off devices, - one take-up device, - induction preheater coil, - guiding system for RM, RMC, RMS conductors, - magnetic separator system in installed in the feeding and handling system for insulating material, - triple crosshead for 3 layers of cable insulating system, - auxiliary caterpillar 2, - length meter counter, - insulation system surface scanner behind the cooling section, - calculating programme, -materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier | ….. - …. (price- currency) | ….. - …. (price- currency) |

- WARRANTY

| Subject matter of the contract | Warranty period (in full months) |
| --- | --- |
| for the development of the documentation, delivery of the equipment and upgrading of the existing EPL 50 421 CCV line. The upgrade shall consist of the following components: - extruder and screw for the semi-conductive screen, - two pay-off devices, - one take-up device, - induction preheater coil, - guiding system for RM, RMC, RMS conductors, - magnetic separator system in installed in the feeding and handling system for insulating material, - triple crosshead for 3 layers of cable insulating system, - auxiliary caterpillar 2, - length meter counter, - insulation system surface scanner behind the cooling section, - calculating programme, -materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier | …. (in months, enter the number of months) |

- TIME OF ORDER DELIVERY

| Subject Matter of the Contract | The expected date of order completion from the moment of signing the contract (in months) (the order completion date should include the entire subject of the contract as described in part I of the request for proposal, i.e. including design, delivery, commissioning and acceptance) |
| --- | --- |
| for the development of the documentation, delivery of the equipment and upgrading of the existing EPL 50 421 CCV line. The upgrade shall consist of the following components: - extruder and screw for the semi-conductive screen, - two pay-off devices, - one take-up device, - induction preheater coil, - guiding system for RM, RMC, RMS conductors, - magnetic separator system in installed in the feeding and handling system for insulating material, - triple crosshead for 3 layers of cable insulating system, - auxiliary caterpillar 2, - length meter counter, - insulation system surface scanner behind the cooling section, - calculating programme, -materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier | - Execution of the upgraded CCV line design ……….……… - New components for upgraded CCV line delivery: ……….……… - Commissioning and final acceptance of the upgraded CCV line ……………. (in months, enter the number of months) |

- TIME OF FAILURE REMOVAL IN TERMS OF ELECTRONICS AND INSTRUMENTATION

| Subject Matter of the Contract | Time of failure removal in terms of electronics and instrumentation (in calendar days) Fulfilment of this criterion is understood as the number of calendar days starting from the reception of notification by the Supplier until the time of restoring full functionality of the equipment |
| --- | --- |
| for the development of the documentation, delivery of the equipment and upgrading of the existing EPL 50 421 CCV line. The upgrade shall consist of the following components: - extruder and screw for the semi-conductive screen, - two pay-off devices, - one take-up device, - induction preheater coil, - guiding system for RM, RMC, RMS conductors, - magnetic separator system in installed in the feeding and handling system for insulating material, - triple crosshead for 3 layers of cable insulating system, - auxiliary caterpillar 2, - length meter counter, - insulation system surface scanner behind the cooling section, - calculating programme, -materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier | …. (enter the number of calendar days) |

- SPECIFY VALIDITY TIME OF THE PROPOSAL

| Validity time of the Proposal (in days) | … (in days, enter the number of days) |
| --- | --- |

- STAFF TRANING

| Subject Matter of the Contract | The price of the Subject Matter of the Contract includes staff training |
| --- | --- |
| for the development of the documentation, delivery of the equipment and upgrading of the existing EPL 50 421 CCV line. The upgrade shall consist of the following components: - extruder and screw for the semi-conductive screen, - two pay-off devices, - one take-up device, - induction preheater coil, - guiding system for RM, RMC, RMS conductors, - magnetic separator system in installed in the feeding and handling system for insulating material, - triple crosshead for 3 layers of cable insulating system, - auxiliary caterpillar 2, - length meter counter, - insulation system surface scanner behind the cooling section, - calculating programme, -materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier | YES NO |

- COUNTRY OF ORIGIN OF THE DELIVERY ITEM

| Subject Matter of the Contract | Country of origin of the Subject Matter of the Contract |
| --- | --- |
| for the development of the documentation, delivery of the equipment and upgrading of the existing EPL 50 421 CCV line. The upgrade shall consist of the following components: - extruder and screw for the semi-conductive screen, - two pay-off devices, - one take-up device, - induction preheater coil, - guiding system for RM, RMC, RMS conductors, - magnetic separator system in installed in the feeding and handling system for insulating material, - triple crosshead for 3 layers of cable insulating system, - auxiliary caterpillar 2, - length meter counter, - insulation system surface scanner behind the cooling section, - calculating programme, -materials necessary for the installation, commissioning and acceptance tests of the line as well as line start-up performed by the representatives of the supplier | … (enter the name of the country) |

- CONSENT FOR PROCESSING PERSONAL DATA:

I hereby declare that:

- I have read the information clause for Tenderers included described below

YES,  NO1

- I give my consent for my personal data to be processed within the scope defined in listed below

YES,  NO2

As far the personal data is concerned, Tele-Fonika Kable S.A informs that the data Administrator is TELE-FONIKA Kable S.A. with its registered office in Myślenice; 32-400, ul. Hipolita Cegielskiego 1 (hereinafter referred to as the “Company”). The administrator can be contacted via the email address: daneosobowe@tfkable.com in all matters concerning processing of personal data and the use of rights in relation to the data processing. Your data will be processed for needs of responding to enquiries submitted by you; the legal basis for data processing is the necessity of implementation of the legitimate interest of the Administrator. A legitimate interest of the Administrator is selling and supporting the sale of its products and services, purchasing and supporting the purchase of goods and services and taking the opportunity to submit information about its services to the customer submitting offers tailored to needs and interests of the customer and increasing the sales of its services. Your personal data may be shared with:

- entities processing the personal data on behalf of the administrator (among others, providers of IT services) – these companies process the data on the basis of an agreement with the Administrator and solely on the instructions from the Administrator;

- entities providing or settling funding from public funds;

- entities providing consulting services, entities providing auditing services;

- other data administrators entitled to obtain data on the basis of applicable law, including the Ministry of Investment and Development as a data administrator in the Central Information System SL 2014.

Your personal data will be stored util the expiry of the storage obligation resulting from the provisions of the law, including applicable regulations on State aid. You have the right to access your data  and to request their rectification, erasure, or limitation of their processing. At your request, the Administrator shall provide a copy of the personal data that are subject to processing, whilst any subsequent copies requested by you may be subject to a fee imposed by the Administrator at a reasonable amount resulting from administrative costs. You have the right to withdraw your consent to processing of your data. Consent withdrawal does not affect the lawfulness of the processing, which had taken place on the basis of the consent given prior to the withdrawal To the extent that your data are processed by automated means with a view to the conclusion and execution of the Contract or data processed on the basis of your consent, you have also have the right to transfer your personal data, namely to obtain your personal data in a structured, widely-used machine readable form. You can also forward the data to another data administrator. You also have the right to issue a complaint to the supervisory authority in charge of personal data protection. In order to exercise these rights, please contact the Data Administrator. The contact details are provided above. Providing personal data for the aforementioned purposes is voluntary. The Tenderer declares that they have read the information clause for Tenderers and give their consent to personal data processing by confirming the above in the Proposal Form that constitutes Annex 2 to this Request for proposal

- REPRESENTATIONS:

- WE DECLARE, that we have read the content of the Request for proposal, explanations and changes to the Request for proposal provided by the Contracting Entity and we deem ourselves bound with the provisions contained therein and the terms and rules of the tender procedure.

- WE ACCEPT the payment terms specified by the Contracting Entity in the Request for proposal

- WE ARE bound by the Proposal for the time period defined in the Request for proposal, however, not shorter than 90 days.

- We undertake to submit performance bond along with all the applicable warranties guarantees specified in the Request for Proposal.

- The devices presented in the Proposal are NEW and COMPLY with all legislation which allows for its use in Poland.

- WE ARE a manufacturer or an authorized representative of the manufacturer of the subject of the contract described in section I of the Request for proposal On the request of the Contracting Entity we shall provide documents that confirm cooperation with the manufacturer that enables us to deliver the subject matter of the contract.

- WE DECLARE that the subject matter is free of legal defects and the interest of third parties.

- WE ATTACH to the Proposal the technical specification of the Subject Matter of the Contract

...................................................

Signature of the Tenderer

Podać cenę wraz z określeniem  waluty w jakiej cena jest wyrażona

Właściwe zaznaczyć

Właściwe zaznaczyć

Właściwe zaznaczyć

Enter price together with the currency in which the price is expressed

Tick the applicable

1 Tick the applicable

2 TIck the applicable